Dev Log – User Testing Core Record

Date: April 08, 2026

Team: Team Sentinel

Work Item: First User Testing - Infrared Sensor Prototype

Traffic Light Status: Yellow

What Shipped: Completed first round user testing & documented all findings

Who Did What: Jane Doe (test design & execution), John Smith (data recording & feedback sorting), Alice Chen (optimization plan drafting)

Accomplishments

Conducted first user testing for infrared sensor prototype in indoor normal/low light environment.

Tested 5 key items with 20 valid trigger attempts; recorded detailed test data and user feedback.

Compiled a complete user testing record sheet and sorted out targeted optimization directions.

Key Test Results

Sensing Success Rate: 90% (18/20), missed trigger at 5m side angle.

Response Latency: 0.8s (normal light), 1.2s (low light).

False/Missed Trigger: 2 false triggers (air flow/heat source), 2 missed triggers (fast movement).

Operation Usability: Parameter adjustment needs tools, no visual feedback.

Scene Adaptability: 92% (security alarm), 85% (corridor fast movement).

Key User Feedback

Sensitive at 1-3m, poor long-distance side-angle detection.

Slow response in low light; occasional false triggers with air flow/heat source.

Inconvenient potentiometer adjustment without visual prompt.

Poor adaptability to fast-moving scenarios (e.g., corridor).

Next Steps

Adjust sensor potentiometer and installation angle to improve long-distance sensitivity.

Add light compensation module & optimize code to shorten low-light response latency.

Increase trigger threshold & adjust algorithm to reduce false/missed triggers.

Replace potentiometer with adjustable button + LED indicator for visual feedback.

Develop scene-specific mode switching for different application scenarios.

Blockers

None

Evidence Attached

Complete Infrared Sensor Prototype - User Testing Record: Internal team document compiled manually; raw data stored in project shared drive. Formal report pending final consolidation.

GitHub link for test data: Test logs are currently in a private repository during the debugging phase; public release scheduled for next sprint.

KANBAN BOARD UPDATE

Project: Infrared Sensor Prototype

Update Date: April 08, 2026

Sprint Status: On track

TO DO

Sensor parameter calibration (sensitivity/delay)

Add light compensation module to hardware

Optimize motion detection algorithm for fast-moving targets

Replace potentiometer with adjustable button + LED indicator

Develop scene-specific mode switching code

Compile user testing feedback into a formal report

IN PROGRESS

Optimize code logic to shorten sensor response latency

Update GitHub with user testing results and optimization plans

DONE

Hardware selection (HC-SR501/HC-SR04 + main control unit)

Schematic drawing & circuit soldering + power-on test

Sensor-microcontroller communication debugging

Basic induction trigger code development

First round user testing (execution & data recording)

Record user testing core findings in Dev Log

Push initial prototype code/files to GitHub